SMART ENVIRONMENT STATION

Software Requirements Specification (SRS)

Functional and Non-Functional Requirements

Student Project Draft | Version 1.0

Prepared: 12 October 2026

# 1. Introduction

The Smart Environment Station is an IoT-based system that monitors environmental conditions. An ESP32 microcontroller collects measurements from connected sensors, including temperature, humidity, and light intensity, and transmits the readings over Wi-Fi to an ASP.NET Core backend. The backend validates and stores readings and provides data to a mobile application. Users can monitor current readings, view historical trends, manage devices, configure thresholds, and receive alerts when configured limits are exceeded.

This document defines proposed functional and non-functional requirements for the student project. Requirements should be reviewed against the final agreed scope and verified through testing.

# 2. Functional Requirements

Functional requirements specify the services and behavior the system must provide. Each requirement has a unique identifier.

| ID | Requirement | Description |
| --- | --- | --- |
| FR-01 | Sensor Data Collection | The system shall collect temperature, humidity, air pressure, and light-intensity readings from the sensors connected to the ESP32. |
| FR-02 | Wireless Communication | The ESP32 shall transmit sensor readings to the backend server over Wi-Fi using the communication protocol selected for the project. |
| FR-03 | Input Validation | The backend shall validate incoming sensor readings and reject or flag invalid payloads before storing them. |
| FR-04 | Data Storage | The system shall store each accepted sensor reading with a timestamp and the identifier of the device that produced it. |
| FR-05 | Current Readings | The mobile application shall display the latest available environmental readings for each device the user is permitted to access. |
| FR-06 | Historical Data | The mobile application shall allow users to select a time period and view historical sensor readings in charts. |
| FR-07 | Device Management | The system shall allow users to view registered devices and perform the device-management actions included in the approved project scope. |
| FR-08 | Threshold Configuration | The system shall allow users to configure threshold values for supported environmental measurements. |
| FR-09 | Alert Generation | The backend shall generate an alert when a valid reading crosses a configured threshold. |
| FR-10 | Alert Delivery | The system shall make generated alerts available to users through the notification mechanism implemented in the project. |

# 3. Non-Functional Requirements

Non-functional requirements define measurable quality attributes and operating constraints. Numerical targets below are proposed acceptance targets, not measured results; the team should validate or revise them during testing.

| ID | Quality Attribute | Description |
| --- | --- | --- |
| NFR-01 | Performance | Under normal test conditions on the local network, the backend shall return a response to a valid sensor-reading request within 2 seconds. |
| NFR-02 | Data Freshness | Under normal local-network conditions, the mobile application shall display a newly processed reading within 3 seconds after the backend has accepted it. |
| NFR-03 | Security | The system shall restrict protected user and device-management operations to authorized users. Credentials and secrets shall not be hard-coded into public source repositories. |
| NFR-04 | Reliability | A temporary Wi-Fi disconnection shall not permanently stop the ESP32 firmware; the device shall attempt to reconnect and resume transmission when connectivity returns. |
| NFR-05 | Data Integrity | Each stored reading shall be associated with the correct device identifier, measurement values, and timestamp. Invalid payloads shall not be stored as valid readings. |
| NFR-06 | Usability | The mobile application shall present readings with clear labels and measurement units and shall distinguish unavailable data from a valid zero value. |
| NFR-07 | Compatibility | The mobile application shall support the operating systems and minimum versions selected by the project team and documented in the final report. |
| NFR-08 | Maintainability | The software shall separate device communication, backend business logic, database access, and mobile presentation into identifiable components. |
| NFR-09 | Local Availability | During a demonstration, the backend shall be accessible to devices on the local network while the host computer, application process, and network remain operational; external Internet access shall not be required. |
| NFR-10 | Resource Efficiency | The ESP32 firmware shall operate within the memory and processing limits of the selected board during continuous monitoring and transmission. |